www.bradyidpal.com

## Instructions

PRODUCT REGISTRATION

Register on-line at www.bradyidpal.com for FREE impact guards and to extend your warranty from six months to one full year.

POWER				Batteries

1

2

Open battery compartment.

- Insert six AA alkaline batteries.

- Close battery compartment.

-or-

### A/C Power

Plug in optional A/C Power Supply.

3

DISPLAY

123

2 GEN S

Text

Line

Label type/

Wiremarker length

Font size  (S, M, L, X, XX)

CREATE LABELS

2

1

3

5

6

- Press

*Press 


General, Banner, 
or Wire/Length
(0.75”, 1.0”, 1.2”, 
1.5”, 2.0”, 2.5”, 3.0”, 
4.0”, 5.0”, 6.0”)

*Use multiple presses to scroll through selections. FUNCTION + key scrolls backwards.

- *Press

7 pt (S) 10 pt (M), 14 pt (L),
20 pt (X), 28 pt (XX)

Enter Text

6

Max. Printable Lines=

General & Wire: 4 Lines

Banner: 2 Lines

Locking Lever must be locked to print. See Change Label Cartridge.

Press

- Cut Label—slide cut lever up

GEN

WIRE

WIRE

BANNER

SERIALIZATION

WIRE1

1 W.75 S

Use arrows   to place cursor under serial character.

- Press  for each consecutive serialization.

WIRE1

1 W.75 S

To produce multiple copies of the same legend, press PRINT before SERIAL .

CHANGE LABEL CARTRIDGE

Unlock Lever

Push Release

Button (on back)

Remove Old

Cartridge

Insert New Cartridge

Lock Lever

2



1

4

KEYPAD

| FUNCTION | PRESS |
| --- | --- |
| Uppercase | +  |
| Lowercase | +  |
| New Line |  |
| Space |  |
| Font Size |  |
| Delete Character |  |
| Clear Legend | + |
| Feed | + |

SYMBOLS

Press  + Character

Prints as

2

CLEANING INSTRUCTIONS

- Remove Label Cartridge.

- Wipe the printhead and platen roller with a cotton swab saturated with isopropyl alcohol.

Cleaning is recommended after each cartridge.

IDENTIFICATION SOLUTIONS & SPECIALTY TAPES

P.O. Box 2131  Milwaukee, Wisconsin 53201-2131 U.S.A.

Phone 1-800-537-8791  www.bradyidpal.com

REV. B 12/01

Y92829